Experiment: 7

Analog Beamforming Analysis Using MATLAB

Aim

To analyze the performance of analog beamforming by studying beamforming gain, beam width,  and main lobe direction using MATLAB

Objective – 1

Analyze Beamforming Gain (dB)

Generate beamforming gain values for different antenna array sizes using MATLAB. Compare  how increasing the number of antenna elements improves the beamforming gain and enhances  signal strength.

MATLAB Code:

clc;

clear;

close all;

antennas = [2 4 8 16 32]; % Number of Antenna Elements

gain = 10*log10(antennas); % Beamforming Gain (dB)

figure

bar(antennas, gain)

grid on

xlabel('Number of Antenna Elements')

ylabel('Beamforming Gain (dB)')

title('Beamforming Gain for Different Antenna Array Sizes')

Expected Output

Objective – 2

Compare Beam Width (Degrees)

Analyze the beam width corresponding to different antenna array sizes using MATLAB. Observe  how increasing the number of antenna elements results in a narrower beam, thereby improving  spatial resolution.

MATLAB Code:

clc;

clear;

close all;

antennas = [2 4 8 16 32]; % Number of Antenna Elements

beamwidth = [90 45 22.5 11.25 5.625]; % Beam Width (degrees)

figure

plot(antennas, beamwidth,'o-','LineWidth',2)

grid on

xlabel('Number of Antenna Elements')

ylabel('Beam Width (Degrees)')

title('Beam Width for Different Antenna Array Sizes')

Objective – 3

Analyze Main Lobe Direction (Degrees)

Simulate beam steering by varying the steering angle of the antenna array using MATLAB.  Observe the change in the main lobe direction and understand how analog beamforming directs  energy toward the intended user.

MATLAB Code:

clc;

clear;

close all;

theta = [-60 -30 0 30 60]; % Main Lobe Direction (degrees)

gain = [1 1 1 1 1]; % Constant normalized gain

figure

polarplot(deg2rad(theta), gain, 'o', 'LineWidth', 2)

title('Main Lobe Direction in Analog Beamforming')

Expected Output

Result

The MATLAB analysis successfully demonstrated the characteristics of analog beamforming. The  results showed that increasing the number of antenna elements improves beamforming gain and  reduces beam width, while phase adjustment enables the main lobe to be steered toward different  directions.